Cuvette Cell Washer

Cuvette Cell Washer 0023-4061

| Potential Hazards while performing these activities: |  |
| --- | --- |
| Slip Hazard | Spilled liquids may cause slip hazard. |
| Fume Hazard | Some solvents used for cleaning cells may generate harmful fumes. |
| Sharps Hazard | Some materials being cleaned with this device may create a sharp hazard if broken. |
| Chemical Exposure Hazard | Splashing in the eyes or on the skin may occur when using this device. |

| Preventions to reduce exposure to hazards: |  |
| --- | --- |
| Eye Protection | Wear safety glasses while operating. |
| Protective Gloves | Wear appropriate gloves for chemical usage. |
| Non-Slip, Closed Shoes | Wear non-slip, closed shoes to avoid spills. |
| Good Ventilation | This activity may require use of a hood if the solvent generates harmful fumes. |
| Read Manual | Become familiar with this guide before operating. |

| Note: - The device materials are resistant to a broad range of chemicals. Use water before trying more aggressive cleaning agents. - Do not mix incompatible materials in the flask. - Keep the cell upright when cleaning, otherwise, the neck of the cell might be broken. - If the cell gets sucked down by the vacuum, remove the funnel to release the vacuum from the cell and then remove the cell. |  |
| --- | --- |
| Setup Steps | Pictures |
| - Ensure a vacuum source tube is connected to the arm on the flask. |  |
| - Install the upper funnel assembly to the flask taking care to firmly seat the seal in the mouth of the flask. |  |
| - Prepare or acquire a desired cleaning solution to be used for the items to be washed. |  |
| Operation | Pictures |
| - Activate the vacuum source by turning the pump on. |  |
| - Place the item to be washed inverted over the center of the platform. |  |
| - Apply a slight downward pressure on the vessel to form a vacuum in the cell. |  |
| - Add cleaning solution to the reservoir until no residue is attached to the cell. |  |
| - Once the flush is complete, repeat twice using roughly 10mL of solvent per rinse. (Ethanol is recommended). |  |
| - Maintain slight downward pressure to quickly dry the vessel. |  |
| - Place rinse solution into an appropriate container and label contents. |  |