Gravity Oven

Gravity Oven

| Potential Hazards while performing these activities: |  |
| --- | --- |
| Electrical Hazard | This equipment contains electrical units that may come in contact with skin or liquids. |
| Heat Hazard | This unit produces heat which creates a hazard if improperly operated. |
| Pinch Hazard | This equipment has a door which may pinch body parts or clothing. |
| Chemical Hazard | Because chemicals are being placed in this unit, chemical hazards are present. |
| Fire Hazard | Any improper use of this equipment may cause a fire hazard. |

| Preventions to reduce exposure to hazards: |  |
| --- | --- |
| Eye Protection | Wear safety glasses while operating. |
| Protective Gloves | Wear appropriate gloves for heat and chemical usage. |
| Non-Slip, Closed Shoes | Wear non-slip, closed shoes to avoid spills. |
| Read Manual | Become familiar with this guide before operating. |

Note: DO NOT place sealed or full samples within unit. DO NOT place explosive, combustible, or flammable materials inside.

| Setup Steps | Pictures |
| --- | --- |
| - Ensure oven is plugged in and on a stable surface. |  |
| - Turn the main power switch on. |  |
| - Turn Overtemperature Control CLOCKWISE to maximum setting. |  |
| - Turn the Main Temperature Dial to desired temp. ALLOW ONE HOUR for temperature to stabilize. |  |
| - Once Main Temperature has STABILIZED, turn the Overtemperature Control Counterclockwise until the safety light comes on. |  |
| - Next, turn the Overtemperature Control CLOCKWISE until light goes out, and then 2 DIVISIONS MORE. |  |
| - Unit is now ready to receive samples. |  |
| Operation | Pictures |
| - Place samples inside of unit. UNIT WILL BE HOT! Please wear appropriate PPE. |  |
| - Ensure door is shut and latch closed. |  |
| - Upon completion. Turn unit power off, and return dials to original settings. |  |
| - After UNIT HAS COOLED, remove samples. |  |